EXP-4

CODE:

clc;

clear;

close all;

Fs = 1000;

Fp = 100;

Fst = 150;

Rp = 1;

Rs = 40;

Wp = Fp/(Fs/2);

Ws = Fst/(Fs/2);

% Butterworth filter

[Nb,Wb] = buttord(Wp,Ws,Rp,Rs);

[b1,a1] = butter(Nb,Wb);

% Chebyshev Type-I filter

[Nc,Wc] = cheb1ord(Wp,Ws,Rp,Rs);

[b2,a2] = cheby1(Nc,Rp,Wc);

% Elliptic filter

[Ne,We] = ellipord(Wp,Ws,Rp,Rs);

[b3,a3] = ellip(Ne,Rp,Rs,We);

% Magnitude responses

[H1,f] = freqz(b1,a1,1024,Fs);

H2 = freqz(b2,a2,1024,Fs);

H3 = freqz(b3,a3,1024,Fs);

plot(f,20*log10(abs(H1)+eps),'b', ...

f,20*log10(abs(H2)+eps),'r', ...

f,20*log10(abs(H3)+eps),'g');

grid on;

xlabel('Frequency (Hz)');

ylabel('Magnitude (dB)');

title('Comparison of IIR Filters');

legend('Butterworth','Chebyshev Type-I','Elliptic');

ylim([-100 5]);

% Display filter orders

fprintf('Butterworth Order = %d\n',Nb);

fprintf('Chebyshev Order = %d\n',Nc);

fprintf('Elliptic Order = %d\n',Ne);

fprintf('Transition Bandwidth = %.1f Hz\n',Fst-Fp);

% Display coefficients

disp('Butterworth Coefficients:');

disp(b1); disp(a1);

disp('Chebyshev Coefficients:');

disp(b2); disp(a2);

disp('Elliptic Coefficients:');

disp(b3); disp(a3);

OUTPUT:

Butterworth Order = 12

Chebyshev Order = 6

Elliptic Order = 4

Transition Bandwidth = 50.0 Hz

Butterworth Coefficients:

1.0e-03 *

0.0002    0.0027    0.0146    0.0488    0.1098    0.1757    0.2050    0.1757    0.1098    0.0488    0.0146    0.0027    0.0002

1.0000   -6.8844   22.4341  -45.5100   63.7725  -64.8501   48.9643  -27.6092   11.5218   -3.4662    0.7128   -0.0899    0.0053

Chebyshev Coefficients:

1.0e-03 *

0.0464    0.2782    0.6956    0.9274    0.6956    0.2782    0.0464

1.0000   -4.8694   10.3807  -12.3367    8.5969   -3.3268    0.5586

Elliptic Coefficients:

0.0197   -0.0171    0.0333   -0.0171    0.0197

1.0000   -3.0330    3.8118   -2.2911    0.5554